Copyright 2018 by The American Institute of Architects (AIA)

Exclusively published and distributed by AVITRU, LLC for the AIA

SECTION 077619 - PORCELAIN-TILE ROOF PAVERS

This Product MasterSpec Section is licensed by AVITRU, LLC to Archatrak Inc. ("Licensee").

This proprietary Product MasterSpec Section uses original MasterSpec text.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Avitru or The American Institute of Architects (AIA). Neither AIA nor Avitru are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

For more information, contact Archatrak Inc., 2 Wisconsin Circle, Suite 700, Chevy Chase, MD 20815; Phone: (866) 206-8316; Website: www.archatrak.com; Email: www.archinfo@archatrak.com.

For information about MasterSpec, contact Avitru at (800) 424-5080 or visit avitru.com.

TIPS:

To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Access Product MasterSpec Sections:

<Double click here to view the list of manufacturer Sections available at productmasterspec.com.>

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Porcelain-tile roof pavers.

          - Paver support systems.

        - Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 321400 "Unit Paving" for brick, asphalt, concrete, and stone pavers on aggregate, mortar, and bituminous setting beds.

      - UNIT PRICES

Retain this article if Work specified in this Section is measured and paid for under the provisions of unit prices. Do not include amounts. Insert descriptions of items in Part 2 or 3 to provide information affecting the cost of the Work that is not included under the unit price.

        - See Section 012200 "Unit Prices" for description of unit prices affecting items specified under this Section.

      - ALTERNATES

Retain this article if products and Work specified in this Section are proposed to be added to or deducted from the base bid amount under the provisions of alternates.

        - See Section 012300 "Alternates" for description of alternates affecting items specified under this Section.

      - ACTION SUBMITTALS

"Action Submittals" are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data: For each type of product.

        - Shop Drawings: For paver tile and paver support system layout, adjacent construction and penetrations, and attachment details, and other conditions.

          - Include data indicating compliance with performance requirements.

Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs below for two-stage Samples.

        - Samples for Initial Selection: For each type of exposed finish, provide representative charts of manufacturer's full range of patterns and colors.

        - Samples for Verification: For each type of exposed finish for each porcelain-tile roof paver selected.

          - Size: Manufacturers' standard size, [4 by 4 inches (102 by 102 mm)] [8 by 12 inches (203 by 305 mm)] [full].

          - Accessories: [Adjustable pedestal support] [rubber paver pad] [and] [shims].

Retain "Delegated-Design Submittal" Paragraph below if design services have been delegated to Contractor.

        - Delegated-Design Submittal: For porcelain-tile roof paver system installation, including analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

MasterSpec includes provisions for LEED 2009, LEED v4, ASHRAE 189.1, IgCC, and Green Globes. Specifiers may wish to insert requirements from sustainability rating systems to improve building performance even when full compliance with a sustainability rating system is not mandated.

Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.

Retain below when using Architrex porcelain pavers.

        - Sustainable Design Submittals:

Architrex porcelain-tile roof pavers contain a considerable percentage of recycled material, to reduce the exploitation of virgin raw materials and the production of waste.

The following are commitments undertaken in order to benefit final users:

- 100 percent recycling of industrial water.

- 100 percent eco-compatible products manufactured using natural raw materials, such as clay, quartz, feldspar, kaolin, and natural dyes, and no harmful substances such as lead and cadmium.

- 100 percent recycling of paver waste deriving from the pre-firing process reduces the consumption of raw materials.

- 100 percent recyclable packaging.

- 100 percent recyclable fired pavers.

          - Product Data: For recycled content, indicating postconsumer and preconsumer recycled content and cost.

      - INFORMATIONAL SUBMITTALS

"Information Submittals" are submittals that do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor. All informational submittals must be reviewed by Architect same as action submittals.

Coordinate "Qualification Statements" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article. If inserting additional entities or specialist, add qualifications to "Quality Assurance" Article.

Architrex does not provide delegated design services.

        - Qualification Statements: For delegated design engineer.

        - Sample Warranty: For warranty.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For porcelain-tile roof pavers and paver support system to include in operation and maintenance manuals.

      - MAINTENANCE MATERIAL SUBMITTALS

        - Extra Stock Material: Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Porcelain-Tile Roof Pavers: Full size units in quantity equal to [5] <Insert number> percent quantity installed for each type, pattern, color, and size.

          - Paver Supports: Units in quantity equal to [5] <Insert number> percent of quantity installed for each type and size. Include manufacturer-furnished adjustment tool.

      - QUALITY ASSURANCE

Retain "Delegated-Design Engineer" Paragraph below to suit Project.

        - Delegated-Design Engineer: A professional engineer who is legally qualified to practice in jurisdiction where Project is located and who is experienced in providing engineering services of the kind indicated.

      - MOCKUPS

        - Mockup: Build mockups to verify selections made under Sample submittals and to set quality standards for materials and execution.

          - Build mockup in size and location indicated on Drawings, or if not indicated, not less than [four] <Insert number> porcelain-tile roof pavers and [nine] <Insert number> roof paver supports.

          - Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - DELIVERY, STORAGE, AND HANDLING

        - Protect products during shipping, handling, and storage to prevent staining, chipping, deterioration of components, or other damage. Store unused materials in original packaging.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Manufacturer's Warranty:

          - Manufacturer and Installer agree to replace:

            - Damaged porcelain-tile roof pavers that deliver to Project site chipped, cracked, or broken prior to being unpackaged.

            - Porcelain-tile roof pavers and paver supports that fail in materials or workmanship within specified warranty period; and when installed and used in accordance with manufacturer's written instructions.

Verify available warranties and warranty periods for porcelain-tile roof paver units and support components.

          - Warranty Period: Three years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Avitru. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - SOURCE LIMITATIONS

        - Obtain porcelain-tile roof pavers and roof paver supports from single source or producer.

      - PORCELAIN-TILE ROOFING PAVERS

Architrex porcelain-tile roof pavers can be installed on all types of roofing membranes.

        - Structural Porcelain-Tile Roof Pavers: Porcelain pavers, abrasion, freeze/thaw, chemical, and stain resistant, for exterior use with paver pedestal support system.

Architrex "Porcelain-Tile Roof Pavers" provide exceptional strength, stain resistance, scratch resistance, frost resistance, and low maintenance.

Besides porcelain pavers, Architrex supplies structural Ipe wood tiles directly sourced from Bolivia and Peru including tiles with wood sourced from FSC-certified forests, fixed-height rubber paver supports for low-height paving, and adjustable-height pedestals for higher elevated paving. For Ipe wood tile information, see www.architrex.com/products/structural-wood-tiles/structural-wood-tiles. For paver pedestal details, see www.architrex.com/products/pedestal-syetem/pedestal-system.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Archatrak Inc.; Porcelain-Tile Roof Pavers or comparable product by one of the following:

            - Ceramiche Caesar.

            - Kronos Ceramiche.

            - Rocersa Ceramica.

            - <Insert manufacturer's name>.

          - Size: [24 by 24 inches (610 by 610 mm)] [24 by 48 inches (610 by 1219 mm)] [48 by 16 inches (1219 by 406 mm)] [36 by 18 inches (914 by 457 mm)] nominal.

          - Thickness: 3/4 inch (19 mm).

In high-wind areas, consider using Architrex galvanized-steel trays and hold-down screws on Eterno™ adjustable-height paver pedestals, or a recommended adhesive to adhere porcelain pavers to pedestals. For additional installation details, see www.architrex.com/products/porcelain-pavers/installation/safety-security.

          - Weight: 9 lb/sq. ft. (44 kg/sq. m).

          - Breaking Strength (ISO 10545-4): 2200 lbf/sq. ft. (1.053 MPa), minimum.

          - Bending Strength (EN 1339): Class 14.

          - Static Load Capacity (EN 12825): 1700 lbf/sq. ft. (0.814 MPa), minimum.

          - Modulus of Rupture (ISO 10545-4): 7000 lbf/sq. in. (0.335 MPa), minimum.

Architrex offers more than 30 colors; see standard colors at www.architrex.com/products/porcelain-pavers/colors and custom colors at www.architrex.com/products/porcelain-pavers/colors-styles-on-demand.

Retain one of five options in "Colors and Textures" Subparagraph below. If retaining first option, indicate colors and textures in a separate schedule.

          - Colors and Textures: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's standard range] [As selected by Architect from manufacturer's custom range] [As selected by Architect from manufacturer's full standard and custom range] <Insert color>.

      - PAVER SUPPORT SYSTEM

Retain this article when pavers are to be installed with a support system. Porcelain pavers may also be installed over sand, gravel, or even grass without using adhesives, grout, or specialized labor. However, due to their exceptional strength, they are especially useful for constructing elevated or rooftop decks, supported only by fixed or adjustable-height pedestals. Pavers may also be installed over 3/4-inch- (19-mm-) high, rubber support pads. For vehicular applications, porcelain pavers can be laid over a reinforced concrete base with thinset mortar and grouted.

Retain "Adjustable-Height-Pedestal Paver Supports" or "Fixed-Height-Pad Paver Supports" Paragraph below to suit Project.

First option in "Adjustable-Height-Pedestal Paver Supports" Paragraph below applies to Architrex' "Eterno™ SE" pedestal. Second option applies to Architrex' "Eterno™ NM" pedestals.

        - Adjustable-Height-Pedestal Paver Supports: Continuously variable, adjustable-height, high-density copolymer polypropylene, screw-jack-type supports. Accommodate height adjustments from [1-1/8 to 21-3/4 inches (28.7 to 552 mm), and slope substrate compensation up to 5 percent with self-leveling "SE" pedestal] [1 to 27-3/4 inches (25.4 to 705 mm), and zero slope substrate compensation and optional use of up to three stackable 1 percent sloping shims with "NM" pedestal], with cushioned SBR-rubber top coating providing sound and shock absorption and built-in removable spacer tabs for consistent spacing for specified pavers,  to provide level paver installation.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Archatrak Inc.; Eterno™ Adjustable Pedestal System or comparable product.

          - Load Capacity: Not less than 2000 lbf (8.9 kN).

          - Paver-Spacer Tabs: Built-in; 5/32 inch (4 mm).

          - Shock Absorption: Rubber-faced head.

        - Fixed-Height-Pad Paver Supports: SBR-rubber cushioning pads of varying thickness with shims and spacers, stackable to accommodate height adjustments from 1/4 to 1-1/2 inches (6.4 to 38 mm), with built-in removable spacer tabs for consistent spacing of specified pavers.

          - Basis-of-Design Product: Subject to compliance with requirements, provide Archatrak Inc.; Fixed Height Paver Supports or comparable product.

          - Load Capacity: Not less than 1500 lbf (6.7 kN).

          - Paver Spacer Tabs: Built-in; 5/32 inch (4 mm).

          - Shims: Manufacturer's standard; 1/16 inch (1.6 mm).

      - PERFORMANCE REQUIREMENTS

        - Product Data: For recycled content, indicating percentages by weight of postconsumer and preconsumer recycled content and cost. Include total weight of products provided.

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

        - Delegated Design: Engage a qualified professional engineer, as defined in Section 014000 "Quality Requirements," to design porcelain-tile roof paver installation.

Retain "Structural Performance" Paragraph below, along with "Delegated Design" Paragraph above, when Contractor is required to take responsibility for structural design of installation. Revise to correspond to requirements of authorities having jurisdiction.

        - Structural Performance: Porcelain-tile roof paver system shall withstand effects of gravity loads and the following loads and stresses, within limits and under conditions indicated:

          - Floors: Uniform load of 125 lbf/sq. ft. (610 kg/sq. m) or a concentrated load of 2000 lbf (8.9 kN), whichever produces greater stress.

          - Walkways and Elevated Platforms Other Than Exits: Uniform load of 60 lbf/sq. ft. (293 kg/sq. m).

          - Walkways and Elevated Platforms Used as Exits: Uniform load of 100 lbf/sq. ft. (488 kg/sq. m).

Retain "All Roof Zones (Corner, Perimeter, and Field-of-Roof) Uplift Pressures" Subparagraph below and delete "Roof Zone 1 (Field-of-Roof) Uplift Pressure," "Roof Zone 2 (Perimeter) Uplift Pressure," and "Roof Zone 3 (Perimeter) Uplift Pressure" Subparagraphs, if roof wind uplift pressures are indicated on Drawings.

For wind uplift information on Architrex porcelain pavers, see www.architrex.com/products/porcelain-pavers/technical-data/wind-uplift.

          - All Roof Zones (Corner, Perimeter, and Field-of-Roof) Uplift Pressures: As indicated on Drawings.

          - Roof Zone 1 (Field-of-Roof) Uplift Pressure: <Insert lbf /sq. ft. (kg/sq. m)>.

          - Roof Zone 2 (Perimeter) Uplift Pressure: <Insert lbf /sq. ft. (kg/sq. m)>, located within <Insert feet (m)> of roof perimeter.

          - Roof Zone 3 (Corners) Uplift Pressure: <Insert lbf /sq. ft. (kg/sq. m)>, located within <Insert feet (m)> of roof outside corner.

For fire resistance information on Architrex porcelain pavers, see www.architrex.com/products/porcelain-pavers/technical-data/fire-resistance.

        - Combustion Characteristics: Porcelain-tile roof paver system complying with Class A flame spread requirements of ASTM E108, when tested by a qualified independent testing laboratory acceptable to authorities having jurisdiction.

      - MATERIALS

        - High-Density Copolymer Polypropylene: Impact, stress-crack, and low-temperature resistant polypropylene complying with ASTM D4101.

        - Porcelain-Tile Roof Pavers: An impervious vitreous tile with a water-absorption rate of 0.5 percent or less as measured by ASTM C373 test method.

        - SBR Rubber: Styrene-butadiene synthetic rubber complying with [ASTM D2000] [ASTM D5605] [ISO 21561-2].

- EXECUTION

      - EXAMINATION

        - Examine substrates and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - Coordinate placement of roof paver supports with roofing section Work.

        - Lay out porcelain-tile roof paver and roof paver support locations, and mark for installation.

      - INSTALLATION OF PAVER SUPPORTS

        - Set paver supports in locations coordinated with approved porcelain-tile roof paver layout. Install in accordance with support manufacturer's written instructions and approved submittals. Adjust paver support heights prior to, and following, installation of porcelain-tile roof pavers. Shim where fine adjustment is necessary using manufacturer-provided shims.

      - INSTALLATION OF ROOF PAVERS

        - Install porcelain-tile roof pavers in accordance with manufacturer's written instructions and approved Shop Drawings. Set porcelain-tile roof pavers in place using placement methods that result in stable installation free from rocking, using sound pavers with no edge or surface damage. Make final in-place level adjustments using manufacturer's furnished tool to adjust paver support heights.

      - CONSTRUCTION TOLERANCES

        - Variation of Level Across Width of Each Porcelain-Tile Roof Paver: 1/16 inch (1.6 mm), maximum.

        - Variation from Level for Paved Areas: Do not exceed 1/4 inch in 10 feet (6.4 mm in 3.0 m), when measured at any location and in any direction.

        - Variation in Joint Width: Do not vary joint thickness more than one-fourth of nominal joint width.

        - Variation in Plane at Joints (Lipping): Do not exceed 1/16-inch (1.6-mm) difference between planes of adjacent units.

        - Variation in Line of Edge at Joints (Lipping): Do not exceed 1/16-inch (1.6-mm) difference between edges of adjacent units, where edge line continues across joint.

      - CLEANING AND PROTECTION

        - Remove and replace loose, chipped, broken, stained or otherwise damaged porcelain-tile roof pavers, or pavers that do not match adjoining units or pattern indicated on Drawings.

        - Cleaning: Remove soiling from exposed porcelain-tile roof paver surfaces; wash and scrub clean. Leave joints between pavers open and clean of debris to allow for proper drainage and airflow.

        - Provide final protection and maintain porcelain-tile roof pavers without damage or deterioration at time of Substantial Completion.

END OF SECTION 077619